www.astekglobe.com

APA012 SERIES  12/15/18W DOE VI adapter

Features

*Universal AC Input Range

*Protections: Short Circuit/Overload/ Overvoltage

*USB socket provided.

*Efficiency: Level VI and V (optional)

*DOE VI/ErP

*Pass LPS/LVD

*100% Hi-Pot & ATE Chroma test

*100% Burn-in test High Ambient Temp(45°C)

*Application: IT/Travel Charger, Communication, Battery charger(CV mode) etc.

Input

Voltage: 90~264VAC

Frequency: 50~60Hz

Input current: 0.4A max.

Inrush current: 40A max

Efficiency Level VI and V (optional)

Output

Over Voltage Protection/Short Circuit Protection: Auto Recovery

Ripple & Noise: 1%

Part No          O/P Voltage    O/P Current(max.)   watt Max

APA012-S03-z      3.3~8V         2.8A              15W

APA012-S12-z      9~17.9V        2.0A              18W

APA012-S18-z      18~30V        1.0A              18W

APA012-S48-z      30.1~48V      0.598A             18W

z: U=US, E=EU, A=Australia/New Zealand, B=British, K=Korean, I=Interchangeable, 08=C8

Load Regulation: 5% Max.

Line Regulation:  1% Max.

Operating Temperature: 0~40degree C

Storage Temperature: -20~85degree C

Storage Humidity: 5~95% RH

Mechanical Type: Fixed and Interchangeable US/EU/UK/AU adapter, Desktop C8 inlet

Safety approvals: DOE VI, cTUVus, TUV-GS, CB,CE, FCC,VCCI

EMC: CE, VCCI, LVD,FCC

Mechanical Drawing

interchangeable adapter                     US plug

EU plug adapter                              UK plug adapter

Desktop C8 inlet